CHAPTER 1

# INTRODUCTION

The fiber optical converter allows the RS232 or RS422 input signals to be converted to fiber optical signals that for long distance, high speed, and high quality communication. There are 3 LED indicators correspond to RxD, CTS, and power signals. A 9V, 500mA-power adapter is provided in the package. To order the fiber optical converter, please select one of RS232 or RS422 version, and select one of DTE or DCE version for signal connection.

- The features of fiber optical converter are:

- Provides RS232 or RS422 signals.

- Provides DTE or DCE mode signals.

- One pair (two converters) is needed for two connected devices.

- Communication distance up to 1500m.

- Communication speeds up to 5Mbps.

- Operation temperature from 0C to 55C.

- ST type fiber optical connector.

- Suitable for three different optical fibers size: 50/125um, 62.5/125um, 100/140um.

- Suitable for multi-mode cable.

- HFBR-2416 fiber optical receiver and HFBR-1414 fiber optical transmitter are used.

- The package contains:

- Two (or one) fiber optical converters.

- 9V, 500mA power adapter.

- User manual.

- DB25 male to DB25 female cable.

CHAPTER 2

# HARDWARE INSTALLATION

Your fiber optical converter is designed to be inserted in the RS232/RS422 port of your computer, please follow the steps listed below.

- Turn off all power to your computer and all devices before installing your fiber optical converter.

- Connect fiber optical converter to RS232/RS422 port.

- Plugs in power adapter to fiber optical converter and connects it to remote site.

- Turn on the computer and power adapter.

CHAPTER 3

# HARDWARE CONFIGURATION

There are one female connector and four fiber optical connectors in the box, the signal assignments are shown below:

- Fiber optical connector

| RTS | CTS | TxD | RxD |
| --- | --- | --- | --- |

- RS232 connector for DTE mode

| Pin # | DB25 Signal Name | RS232 Name | Signal Direction |
| --- | --- | --- | --- |
| 1 | Chassis Ground (GND) | AA | Common |
| 2 | Transmit Data (TxD) | BA | Output |
| 3 | Receive Data (RxD) | BB | Input |
| 4 | Request to Send (RTS) | CA | Output |
| 5 | Clear to Send (CTS) | CB | Input |

- RS232 connector for DCE mode

| Pin # | DB25 Signal Name | RS232 Name | Signal Direction |
| --- | --- | --- | --- |
| 1 | Chassis Ground (GND) | AA | Common |
| 2 | Receive Data (RxD) | BB | Input |
| 3 | Transmit Data (TxD) | BA | Output |
| 4 | Clear to Send (CTS) | CB | Input |
| 5 | Request to Send (RTS) | CA | Output |

- RS422 connector for DTE mode

| Pin # | DB25 Signal Name | Signal Direction |
| --- | --- | --- |
| 2 | Transmit Data (TxD+) | Output |
| 3 | Receive Data (RxD+) | Input |
| 4 | Request to Send (RTS+) | Output |
| 5 | Clear to Send (CTS+) | Input |
| 14 | Transmit Data (TXD-) | Output |
| 15 | Receive Data (RxD-) | Input |
| 16 | Request to Send (RTS-) | Output |
| 17 | Clear to Send (CTS-) | Input |

- RS422 connector for DCE mode

| Pin # | DB25 Signal Name | Signal Direction |
| --- | --- | --- |
| 2 | Receive Data (RxD+) | Input |
| 3 | Transmit Data (TxD+) | Output |
| 4 | Clear to Send (CTS+) | Input |
| 5 | Request to Send (RTS+) | Output |
| 14 | Receive Data (RxD-) | Input |
| 15 | Transmit Data (TxD-) | Output |
| 16 | Clear to Send (CTS-) | Input |
| 17 | Request to Send (RTS-) | Output |

To use the fiber optical converter, you must connect it with four fiber optical cables, then connect both DB-25 connectors to PC or devices (we call Device in the follow). Please note that, you must use DCE or DTE mode correctly, otherwise it can not work. Suppose the Device is DTE mode, you must set the converter to DCE, otherwise if the Device is DCE mode, you must set the converter to DTE.

In the following, we will introduce how to set the hardware configuration in different modes, and also show the signal flow of both fiber optical converters. It is clearly that only connect transmit signal to receive signal and connect request to send signal to clear to send signal.

- Both DTE modes

TxD 2---------------------------1 RxD

RxD 1---------------------------2 TxD

Device  -------- RTS 4---------------------------3 CTS  -------- Device

CTS 3---------------------------4 RTS

| DCE mode | DTE mode | Fiber Optical Cables | DTE mode | DCE mode |
| --- | --- | --- | --- | --- |
| Device side | Converter side | Converter side | Converter side | Device side |

- One DTE mode and one DCE mode

TxD 2---------------------------1 RxD

RxD 1---------------------------2 TxD

Device  --------  RTS 4---------------------------3 CTS  -------- Device

CTS 3---------------------------4 RTS

| DTE mode | DCE mode | Fiber Optical Cables | DTE mode | DCE mode |
| --- | --- | --- | --- | --- |
| Device side | Converter side | Converter side | Converter side | Device side |

- Both DCE modes

TxD 2---------------------------1 RxD

RxD 1---------------------------2 TxD

Device  --------  RTS 4---------------------------3 CTS  -------- Device

CTS 3---------------------------4 RTS

| DTE mode | DCE mode | Fiber Optical Cables | DCE mode | DTE mode |
| --- | --- | --- | --- | --- |
| Device side | Converter side | Converter side | Converter side | Device side |

RS232 BOX

| RS-232 (J1,J2,J3,J4) |  |  |  |  |  | DC 9V 500mA |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCE |  |  |  |  | 1 – 2 SHORT |  |  |  |  |
| DTE |  |  |  |  | 2 – 3 SHORT |  |  |  |  |
| #### RS – 232 PIN ASSIGNMENT |  |  |  |  |  |  |  |  |  |
| 1 | 2 | 3 | 4 |  |  | 5 | 6 | 7 | 20 |
| GND | TX | RX | RTS |  |  | CTS | DSR | GND | DTR |
| DTR, DSR |  |  |  | JIO 1 – 2 SHORT |  |  |  |  |  |

RS422 BOX

| RS-422 (J6) |  |  |  |  |  | DC 9V 500mA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCE |  |  |  |  | 1 – 2 SHORT |  |  |  |  |  |  |
| DTE |  |  |  |  | 2 – 3 SHORT |  |  |  |  |  |  |
| #### RS – 422 PIN ASSIGNMENT |  |  |  |  |  |  |  |  |  |  |  |
| 1 | 2 | 3 | 4 | 5 |  |  | 7 | 14 | 15 | 16 | 17 |
| GND | TX+ | RX+ | RTS+ | CTS+ |  |  | GND | TX- | RX- | RTS- | CTS- |

APPENDIX A

# WARRANTY INFORMATION

## A.1  Copyright

Copyright 1997, 1998 DECISION COMPUTER INTERNATIONAL CO., LTD. All rights reserved. No part of FIBER OPTICAL hardware and manual may be reproduced, transmitted, transcribed, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, chemical, manual, or otherwise, without the prior written permission of DECISION COMPUTER INTERNATIONAL CO., LTD.

Each piece of FIBER OPTICAL package permits user to use FIBER OPTICAL only on a single computer, a registered user may use the program on a different computer, but may not use the program on more than one computer at the same time.

Corporate licensing agreements allow duplication and distribution of specific number of copies within the licensed institution. Duplication of multiple copies is not allowed except through execution of a licensing agreement. Welcome call for details.

## A.2 Warranty Information

DECISION warrants that for a period of one year from the date of purchase (unless otherwise specified in the warranty card) that the goods supplied will perform according to the specifications defined in the user manual. Furthermore that the FIBER OPTICAL product will be supplied free from defects in materials and workmanship and be fully functional under normal usage.

In the event of the failure of a FIBER OPTICAL product within the specified warranty period, DECISION will, at its option, replace or repair the item at no additional charge. This limited warranty does not cover damage resulting from incorrect use, electrical interference, accident, or modification of the product.

All goods returned for warranty repair must have the serial number intact.  Goods without serial numbers attached will not be covered by the warranty.

Transportation costs for goods returned must be paid by the purchaser. Repaired goods will be dispatched at the expense of FIBER OPTICAL.

To ensure that your FIBER OPTICAL product is covered by the warranty provisions, it is necessary that you return the Warranty card.

Under this Limited Warranty, DECISION's obligations will be limited to repair or replacement only, of goods found to be defective as specified above during the warranty period. DECISION is not liable to the purchaser for any damages or losses of any kind, through the use of, or inability to use, the FIBER OPTICAL product.

DECISION reserves the right to determine what constitutes warranty repair or replacement.

Return Authorization: It is necessary that any returned goods are clearly marked with an RA number that has been issued by DECISION. Goods returned without this authorization will not be attended to.

APPENDIX B

# DATA SHEET