CHAPTER 1

# INTRODUCTION

The PCCOM PCI bus 2 port RS422/RS485 with isolator adapter is an 32 bits PCI bus board with Plug and Play (PnP) features, it provides two asynchronous serial communication ports (RS422/RS485), which link the computer and serial peripheral devices such as terminals, modems, serial printers, plotters, ... etc. The on board isolator can isolate   voltage up to 2500V r.m.s..

The PCCOM PCI bus 2 port adapter is particularly suited to facilitate the connection of terminals (VDUs) in multi-user operating systems. The PCCOM board may be installed in any Pentium or hardware compatible systems. The PnP features let hardware configuration for IRQ and I/O address is detected by BIOS automatically, you don’t need set switch and jumper.

Since most of the computer has its own COM1 and COM2, the PCCOM PCI bus 2 port adapter can be configured from COM4.

On board 16550 chip provides two 16550 functions which contains 16 bytes FIFO buffer for each 16550 ports. There are two kinds of mode can be choose, one is normal speed mode that its baud rate up to 115200, another is high speed mode that its baud rate up to 460K.

- The features of the PCCOM PCI bus 2 port RS422/RS485 adapter are:

- 32 bit PCI bus with Plug and Play (PnP) features.

- Two RS422/RS485 ports for asynchronous communications.

- Provides isolator function, which can isolate voltage up to 2500V r.m.s.

- Suitable for SCO UNIX, Linux, MS/DOS, WINDOWS NT/2000/XP, WINDOWS 95/98/ME, OS/2... etc.

- Pentium hardware compatibles.

- Baud rate up to 115200 for normal speed mode and up to 460K for high-speed mode.

- Provides 16550 port that contains 16 bytes FIFO for each port.

- Up to 4 boards be installed in one computer system.

- Software compatible with PCCOM98/2000.

- Operating temperature 0 to 60 ºC.

- Storage temperature -20 to 70 degree °C.

- Humidity 5% to 95% in non-condensing.

CHAPTER 2

# UNPACKING INFORMATION

- Check that your PCCOM package includes the following items:

- PCCOM PCI bus 2 RS422/RS485 with isolator adapter.

- User manual.

- Decision Studio CD for PCCOM software.

- Warranty form.

CHAPTER 3

# SYSTEM REQUIREMENTS

- Before installing your PCCOM PCI bus 2 port adapter, make sure that:

- The host computer is a Pentium or its compatibles.

- The switch and the jumpers are properly configured.

- The operating system you intend to use is capable of driving multiple serial ports.

CHAPTER 4

# HARDWARE INSTALLATION

Your PCCOM PCI bus 2 port adapter is designed to be inserted in any available PCI slot in your Pentium or compatibles. You must plug this board to your computer before installing PCCOM software. In order to gain access to the expansion slots, follow the steps listed below:

- Turn off all power to your computer and all peripheral devices before installing your PCCOM PCI bus 2 port adapter.

- Remove the cover of the computer.

- Insert the PCCOM PCI bus 2 port adapter into any available PCI slot. Make sure the adapter is firmly seated in the chosen slot.

- Replace the cover of the computer.

- Connect cables to DB9 connectors as required.

- Turn on the power of your computer.

CHAPTER 5

# SWITCH AND JUMPER SETTING

The Plug and Play features let hardware configuration for IRQ and I/O address is detected by BIOS automatically, so that it need not switch and jumper setting. However the on board switches can be used to identify card number by the users (if you do not set the switch, the PnP BIOS will assign card number automatically).

JP1 (Jumper 1)

Select RS422 or RS485 mode for port 1.

JP2 (Jumper 2)

Select receive mode for port 1.

JP3 (Jumper 3)

Select RS422 or RS485 mode for port 2.

JP4 (Jumper 4)

Select receive mode for port 1.

JP5 (Jumper 5)

Select high-speed mode or normal speed mode for port 1.

JP6 (Jumper 6)

Select high-speed mode or normal speed mode for port 1.

SW1 (Switch 1)

Identifies card number.

1. Select RS422 or RS485 Mode

JP1                    JP3

The JP1 is used to select RS422 or RS485 for port 1 and the JP3 is used to select RS422 or RS485 for port 2. To short pin 1,2 means select RS422 mode, otherwise to short pin 2,3 means select RS485 mode.

| Jumper | RS422 | RS485 |
| --- | --- | --- |
| JP1 for port 1 | Short 1,2 | Short 2,3 |
| JP3 for port 2 | Short 1,2 | Short 2,3 |

2. Select Receive Mode

JP2                    JP4

The JP2 and JP4 are used to select receive mode. To short 1,2 means select receive always enable, and short 2,3 means select RTS to enable receive. The JP2 is used to select receive mode for port 1 and the JP4 is used to select receive mode for port 2.

| Jumper | Receive always enable | Enable by RTS |
| --- | --- | --- |
| JP2 for port 1 | Short 1,2 | Short 2,3 |
| JP4 for port 2 | Short 1,2 | Short 2,3 |

3. Select High Speed or Normal Speed Mode

JP5                    JP6

The JP5 and JP6 are used to select high speed mode or normal speed mode; the clock is 7.3728MHZ for high-speed mode, and 1.8432MHZ for normal speed mode. For high-speed mode, the baud rate speed up to 460K. The JP5 is used to select speed for port 1 and the JP6 is used to select speed for port 2.

| Jumper | Normal Speed | High Speed |
| --- | --- | --- |
| ##### JP5 for port 1 | Short 1,2 | Short 2,3 |
| JP6 for port 2 | Short 1,2 | Short 2,3 |

| Pin | Clock Rate | Mode |
| --- | --- | --- |
| Short 1,2 | 1.8432MHZ | Normal Speed |
| Short 2,3 | 7.3728MHZ | High Speed |

4. Card Identifier

The switch is used to identify card number, default setting is card 15. There are two methods to set the card number:

- PnP mode

Just plug in PCCOM PCI bus 2 port adapter into PCI slot, the PCI BIOS will allocate I/O address to each adapter automatically and assign card number start from 0 to each adapter. You may set any card number at PnP mode, and you need use software tools to distinguish port id. Almost all of the operating systems run at PnP mode.

- manual mode

Set card number by card identifier switch, the PCI BIOS will assign pre-allocated I/O address to each adapter. Please set different card number to each adapter (do not duplicate card number setting).

| 1 | 2 | 3 | 4 | Card Number |
| --- | --- | --- | --- | --- |
| OFF | OFF | OFF | OFF | 15 |
| ON | OFF | OFF | OFF | 14 |
| OFF | ON | OFF | OFF | 13 |
| ON | ON | OFF | OFF | 12 |
| OFF | OFF | ON | OFF | 11 |
| ON | OFF | ON | OFF | 10 |
| OFF | ON | ON | OFF | 9 |
| ON | ON | ON | OFF | 8 |
| OFF | OFF | OFF | ON | 7 |
| ON | OFF | OFF | ON | 6 |
| OFF | ON | OFF | ON | 5 |
| ON | ON | OFF | ON | 4 |
| OFF | OFF | ON | ON | 3 |
| ON | OFF | ON | ON | 2 |
| OFF | ON | ON | ON | 1 |
| ON | ON | ON | ON | 0 |

- The card number starts from 0 to 15.

CHAPTER 6

# RS422/RS485 CABLING INFORMATION

## 6.1 RS422 Cabling Information for DB9

The communication interface follows the EIA RS422 standard.  The signal assignments for a standard DB9 connector are shown below:

| Pin | Description |
| --- | --- |
| 1 | Chassis Ground (GND) |
| 4 | Transmit Data+(TxD+) |
| 5 | Transmit Data-(TxD-) |
| 8 | Receive Data+(RxD+) |
| 9 | Receive Data-(RxD-) |

To connect the RS422 to other DATA TERMINAL EQUIPMENT (DTE) devices, the developers recommend using a DTE to DTE connection as shown below.

| HOST | REMOTE |
| --- | --- |
| Chassis Ground | Chassis Ground |
| Transmit Data(+) | Receive Data(+) |
| Receive Data(+) | Transmit Data(+) |
| Transmit Data(-) | Receive Data(-) |
| Receive Data(-) | Transmit Data(-) |

## 6.2 RS485 Cabling Information for DB9

The RS485 signal assignment is shown as follows.

| Pin | Description |
| --- | --- |
| 1 | Signal Ground |
| 4 | Transmit Data+(TxD+) |
| 5 | Transmit Data-(TxD-) |
| 8 | Receive Data+(RxD+) |
| 9 | Receive Data-(RxD-) |

The RS485 communication is based on cable sharing method, which is connected as following:

| Computer at site 1 | Computer at site 2 |
| --- | --- |
| TRD+ | TRD+ |
| TRD– | TRD– |
| Ground | Ground |

Where TRD+ means short pin 4 and pin 8 at connector side, and TRD- means short pin 5 and pin 9 at connector side. You must short both pins by yourself.

## 6.3 RS422 Cabling Information for DB25

The communication interface follows the EIA RS422 standard.  The signal assignments for a standard DB25 connector are shown below:

| Pin | Description |
| --- | --- |
| 1 | Transmit Data-(TxD-) |
| 5 | Receive Data+(RxD+) |
| 7 | Chassis Ground (GND) |
| 20 | Transmit Data+(TxD+) |
| 22 | Receive Data-(RxD-) |

APPENDIX A

# PC COM DIAGNOSTIC UNDER MS/DOS

The TESTCOM is a diagnostic program, provide routines to test your PCCOM PCI 2 port serial adapter under MS-DOS. It has both internal and external loop back tests. During external loop back test, a loop back plug must be connected to each port being tested. You can also select different hardware settings during testing from the SETUP RS232/RS422 menu.

User can get TESTCOM program from Decision Studio CD.

- To test your PCCOM 2 port adapter under MS/DOS, please type

TESTCOM

The TestCom menu will appear.

Setup RS232/RS422 – is used to setup the baud rate etc.

Internal loop test – is used to test the IC.

External loop test – is used to test the ports.

Auto – is used to test the IC and the ports of the card.

APPENDIX B

# PC COM DEVICE DRIVER FOR MS/DOS

## B.1 PCCOM Software

The PCCOM software is a high performance, easy to use RS232/RS422 device driver for PC/486, Pentium or compatibles. Under MS/DOS environment, you can set up your serial ports by PCCOM device driver, and these serial ports can be treated as COM1: and COM2: devices. The setup procedure provides flexible functions to specify the configuration of multi-serial card, that is, the hardware configurations of I/O port number, I/O port address, interrupt and interrupt vector are user selectable.

After the device driver is installed, It takes over communication between CPU and multi-serial cards such as four port card, eight port card, ... etc. For each I/O port, the service routine handles a ring buffer to keep track of all I/O data. Moreover, the PCCOM software provides library routines (C, PASCAL, BASIC, FoxPro) and DOS communication interface (DOS device driver, BIOS call) for several access levels.

- For more details, please refer PCCOM manual.

B.2 Software Installation

When the board is installed, please install software drivers as follows:

STEP 1: Prepare PCI.OPT file

The PCI.OPT file contents are :

/B:4 /D:COM3 /P:[:2:(2k:9600:N-8-1:RTS+DTR:XON) * 2]

STEP 2: Prepare CONFIG.SYS file

Insert statement into CONFIG.SYS file

For normal speed

DEVICE = PCCOM.SYS @c:\pci.opt

For high speed

DEVICE = PCCOMH.SYS @c:\pci.opt

The syntax of PnP function is

/P:[Card_id:Portno:Port]

The Card_id field is defined as 1 or 2 or 3 or 4 or nil, if you use PnP mode, just fill nil in card_id field that like step 1, otherwise you can specify card_id to match the card identifier switch. The Portno field is used to specify number of ports in this adapter. The Port field is defined as PCCOM V2.0.

- If more than one PCCOM board is installed, Please repeat directive /P of the step 1.

APPENDIX C

# UNIX CONFIGURATION

The distribution CD contains SCO UNIX driver. Our drivers also provide transparent printer features that let user to connect local printer from auxiliary port of terminal. The PCI BIOS will assign card number automatically when you use PnP method, otherwise you can use card identifier switch to set your card number. The software installation procedures are shown is the bellows.

## C.1 Hardware Configuration

- First adapter

| Port | Device Name | MODEM Name | Transparent Printer Name |
| --- | --- | --- | --- |
| 1 | /dev/ttyj11 | /dev/ttyJ11 | /dev/lpj11 |
| 2 | /dev/ttyj12 | /dev/ttyJ12 | /dev/lpj12 |

- Second Adapter

| Port | Device Name | MODEM Name | Transparent Printer Name |
| --- | --- | --- | --- |
| 1 | /dev/ttyj21 | /dev/ttyJ21 | /dev/lpj21 |
| 2 | /dev/ttyj22 | /dev/ttyJ22 | /dev/lpj22 |

- Third Adapter

| Port | Device Name | MODEM Name | Transparent Printer Name |
| --- | --- | --- | --- |
| 1 | /dev/ttyj31 | /dev/ttyJ31 | /dev/lpj31 |
| 2 | /dev/ttyj32 | /dev/ttyJ32 | /dev/lpj32 |

- Fourth Adapter

| Port | Device Name | MODEM Name | Transparent Printer Name |
| --- | --- | --- | --- |
| 1 | /dev/ttyj41 | /dev/ttyJ41 | /dev/lpj41 |
| 2 | /dev/ttyj42 | /dev/ttyJ42 | /dev/lpj42 |

## C.2 Software Installation

- The installation procedure for the device drivers is described as follows:

- Login as a root user.

- Insert distribution CD (which contains device drivers) into CD-ROM drive d:, then copy the files from the distribution CD to a temporary directory.

#cd /

# doscp d:dc.tz ./dc.tar.Z

# zcat dc.tar / tar xvfp -

- To install device drivers, please type:

#cd /usr/sys/pccom/dc

# ./install

- Reboot the system. Now, your new UNIX system that includes device drivers is activated.

- Enable each terminal by using the enable command.

# enable   ttyj11

# enable   ttyj12

.

.

- Connect each terminal to connector.

NOTE:

- If the new system fails to reboot, please boot the original system. When system is boot, please press return key to halt autoboot, then type

:unix.old

- To remove device driver from UNIX, please type

- login as a root user

- # cd /usr/sys/pccom/dc

- Remove PCCOM Driver from the kernel

#./ remove

- After installation, please enable each port by enable command and disable port by disable command.

#disable ttyj11

- To change baud rate, please update /etc/inittab and /etc/conf/cf.d/init.base files.

## C.3 Option for High  Speed

The configuration of High-Speed Baud Rate card is change as follows:

| Original | Extensible |
| --- | --- |
| 50 | 14.4 K |
| 75 | 28.8 K |
| 110 | 57.6 K |
| 134 | 76.8 K |
| 150 | 115.2 K |
| 200 | 153.6 K |
| 300 | 230.4 K |
| 600 | 460.8 K |
| 1200 | 1200 (unchanged) |
| 2400 | 2400 (unchanged) |
| 4800 | 4800 (unchanged) |
| 9600 | 9600 (unchanged) |
| EXTA | 19200 (unchanged) |
| EXTB | 38400 (unchanged) |

## C.4 Transparent Printer

The default device names to Transparent Printer(TP) are /dev/lpXYY, that is, the prefix name is changed from "tty" to "lp" but the other "XYY" is the same. e.g. under default device names, the corresponding TTY line of /dev/lpj11 is /dev/ttyj11.

By multiplexing a serial line, there are two sorts of data channels for TTY data(by /dev/ttyXYY) and TP data(by /dev/lpXYY). If the /dev/ttyj11 is used for a TTY, it has to be enabled before you would like to print data through /dev/lpj11 to a printer that connected to the terminal that is operated via /dev/ttyj11.

The channel for TP data that is uni-directional is used to transmit the data from a host to a terminal only. The differentiates of TTY data and TP data in the same serial line is that TP data are encapsulated within a couple of PRINT-ON and PRINT-OFF escape strings that are recognized by connected terminals. The PRINT-ON and PRINT-OFF is defined by connected terminals.

The scheme to multiplex a serial line for these two channels is based on time-division method. The time slices for TTY or TP data are generated according to the entry procedure, polling, in the PCCOM driver, which is periodically called by system clock. The period of system clocks is different among various operating systems, e.g. most UNIXs is 100hz, but SCO Xenix is 50hz.

The interval reserved for TTY or TP channel in the same serial line is important to output TP data to a low-speed printer through high-throughput line from PCCOM cards if there is no flow control XON/XOFF to the serial line.

The lpx command is used to adjust the time interval for TTY or TP data and the TP protocol.

lpx [option] device name

- option:

-t number: set interval for TTY -l number: set interval for Transparent Printer -n string: set esc string to turn on printer -f string: set esc string to turn off printer -T : get interval for TTY -L : get interval for Transparent Printer -N : get esc_string to turn on printer -F : get esc_string to turn off printer

- device_name : lpXYY

The range of interval reserved for TTY or TP channel is from 1 to maximum integer. The default setting for any /dev/lpXYY is as follows:

Interval for TTY : 50 Interval for TP : 1 PRINT - ON escape : “\033[5i” (ESC[5i) PRINT – OFF escape : “\033[4i” (ECS[4i)

- The examples to invoke lpx

- Set 60 time slices reserved for /dev/ttyj11

# lpx -t 60 /dev/ttyj11

- Set 2 time slices reserved for /dev/lpj11

# lpx –12 /dev/lpj11

- Get the time slices reserved for /dev/lpj11

# lpx –L /dev/lpj11

- Set PRINT-ON string for /dev/lpj11

# lpx –n “\033[51” /dev/lpj11

- Get PRINT-OFF string for /dev/lpj11

# lpx –F /dev/lpj11 \033[4i

APPENDIX D

# WINDOWS95/98 CONFIGURATION

Welcome to the Decision PCCOM PCI cards series. This series enables you to utilize the built in plug and play functionality of Windows 95/98, combined with the power of PCI based serial communication.

## D.1 Installation

- For optimum PnP functionality, please install one card at a time.

- Unplug your PC, open it and install one Decision PCCOM PCI card.

- Start your computer and wait until Windows 95/98 is loaded.

- Windows 95/98 will detect the new PCI card that is installed in your computer, and will prompt you for a proper driver.

- Insert the device driver CD into your computer, and make sure Windows 95/98 finds the drivers by eventually browsing through the directory structure of the CD.

- After installing the core device drivers for the PCI card, Windows 95/98 will detect and install software for each COM port on the card. This may take 5 to 10 seconds per port.

- After Windows 95/98 has detected and configured all ports, you may begin using the PCI card. To verify that the installation process completed successfully, please proceed into the Control Panel / System / Device Manager.

- Locate the additional COM ports in the ports section.

- If you need install more than one card, please run above procedure again. Never try to install 2 or more cards at the same time for you will have errors in installation.

## D.2 Remove Ports

- Go to [Control Panel Applet]\[System]\[Device Manager]\[Ports].

- Select the port you want to remove then press “Remove” to delete specified port. Reboot for changes to take effect.

APPENDIX E

# WINDOWS2000 CONFIGURATION

Welcome to the Decision PCCOM PCI cards series. This series enables you to utilize the built in plug and play functionality of Windows 2000, combined with the power of PCI based serial communication. During the boot-up of Windows 2000 the hardware will be automatically detected by the WINDOWS 2000, just ignore this dialog box by clicking cancel and during login, use the administrator user name for installation

## E.1 Installation

- For optimum PnP functionality, please install one card at a time.

- Unplug your PC, open it and install one Decision PCCOM PCI card.

- Start your computer and wait until Windows 2000 is loaded.

- Windows 2000 will detect the new PCI card that is installed in your computer, and will prompt you for a proper driver.

- Insert the device driver CD into your computer, and make sure Windows 2000 finds the drivers by eventually browsing through the directory structure of the CD.

- After installing the core device drivers for the PCI card, Windows 2000 will detect and install software for each COM port on the card.

- After Windows 2000 has detected and configured all ports, you may begin using the PCI card. Inside the Control Panel, you will see the Icon of Decision PCCOM Adapters, indicating that the driver has been successfully installed..

- Double click the Icon Decision PCCOM Adapters, another window will appear on screen and will display an installed card with corresponding COM port.

- If you need install more than one card, please run above procedure again. Never try to install 2 or more cards at the same time for you will have errors in installation.

## E.2 Remove Ports

  - Go to [Control Panel Applet]\[Decision PCCOM Adapters].

  - Select the port you want to remove then press “Remove Port” to delete specified port. Reboot for changes to take effect.

APPENDIX F

# WINDOWS/NT CONFIGURATION V3.5 up

Welcome to the Decision PCCOM PCI cards series. This series enables you to utilize the built in plug and play functionality of Windows NT, combined with the power of PCI based serial communication.

## F.1 Installation

- For optimum PnP functionality, please install one card at a time.

- Unplug your PC, open it and install one Decision PCCOM PCI card.

- Start your computer and wait until Windows NT is loaded.

- Insert the device driver CD into your computer. The CD is auto run, so you just wait until Decision Studio Applet appears.

- Click “Device Drive” button, “Windows Operating System” then a selection of windows operating system platform appears.

- Click “Windows NT” |button to view the device list of different hardware products.

- Select “PCCOM Multi Port Serial Card”. Install Shield will do installation, for you just click “Next” When windows prompts you to resume installation procedure.

- To configure, go to [Control Panel]\[Decision PCCOM Adapters]. Click the “+” of the card you want configure, then select which COM port you want to set.

## F.2 Remove Ports

Go to [Control Panel]\[Ports] then select what you want to remove and click “Delete” to remove ports.

## F.3 Uninstall

Enter [Control Panel]\[Decision PCCOM Adapter], and click [Completely Remove Driver].

APPENDIX G

# LINUX CONFIGURATION

The PCCOM PCI bus 2 port adapter can be installed in the Linux by using serial device driver supported by Linux. For more details, please refer to 'setserial' man-pages.

- Please uncomment the appropriate lines in /etc/rc.d/rc.serial to enable auto-configuration of PCCOM PCI bus 2 port card.

For example, uncomment the following lines for PCCOM PCI bus 2 port card.

${SETSERIAL} /dev/cua4 ${AUTO_IRQ}autoconfig${STD_FLAGS} ${SETSERIAL} /dev/cua5 ${AUTO_IRQ}autoconfig${STD_FLAGS}

APPENDIX H

# DIAGNOSTIC UNDER WINDOWS

After installing the PCCOM PCI bus 2 port adapter to your computer, you can test it if it is functioning correctly via HyperTerminal (a Windows package communication program), or by using the Decision Terminal (a software of the Decision Computer International Co. Ltd.) included on the CD.  Just make sure that there is a loop back plug connected to the COM Port under test.

## H.1  Using Hyper Terminal

To test the card using the Hyper Terminal. Please do the following steps:

- Run the HyperTerminal program.

- During connection, the program will ask you to enter your name and choose an icon for the connection. Enter any name and select any icon.

- After entering your name and selecting icon you will be ask for country code, area code, phone number and connect using what. Ignore all edit box except for the connect using, click the combo box and select Direct to COMx (You can use any port but usually the COM1 and COM2 is used by the computer motherboard).

- Put the proper COM x properties.

- You can now begin to type any message. Take note that whatever you type must appear to the textbox as long as you have a loop back on the COM port of your card!

## H.2  Using Decision Terminal

To test the card using the Decision Terminal. Please do the following steps:

- Install the software using the Decision Studio (It is inside the CD AutoOpen program).

- Run the program (Decision Terminal) at the program menu of the start menu.

- Setup the COM port properties by selecting the setup option on the File menu.

- You can now begin to type any message to transmit textbox. Take note that whatever you type must appear to receive textbox as long as you have a loop back on the COM port of your card.

- You can also click the Test button to test your card automatically.

APPENDIX I

# SOFTWARE DEVELOPMENT INFORMATION

In the Decision Studio CD, it contains PCCOM device driver for WINDOWS 3.1/95/98/2000/NT/ME/XP, DOS, OS/2, UNIX, Linux, and the development tools for serial communication and telecommunication.

## I.1 PCCOM Professional

The PCCOM Professional is the Serial Communication Software Development Tool Kits Under Windows 95/98/2000/NT/ME/XP. It contains Complete OCX and DLL Microsoft Win32 API software accessory function for application software developer. It is an easy to use tool and speed up serial communication application software development time and cost. Wide compatibility range hardware support for PCCOM multiport serial card and any standard port, and support Zmodem and Kermit file transfer protocols for ASCII, Text and Executable file.

## I.2 PCCOM RemoteCom

The PCCOM RemoteCom is an OCX programming development tool for made application software. That will use to enhance RS-232/422 serial communication through Internet or Intranet by stand-alone program or by browser through World Wide Web (WWW) under Windows 95/98/2000/NT/ME/XP. User may call OCX functions to communicate with RemoteCOM both Server and Client using Internet communication; or encapsulate OCX function and remote serial ActiveX Control, then run development application program under Internet browser (IE and Netscape).

## I.3 PCCOM TeleCom

The PCCOM TeleCom is an OCX software accessory suitable for software engineer to develop application program under Windows 95/98/2000/NT/ME/XP environment. It really makes computer and communication application combine together. Wide compatibility range hardware support for PCCOM multiport serial card and any standard port. The PCCOM TeleCom software kit supports five major telecommunication functions, which can be used to

- Send message to “PAGER”(BB Call).

- Send and receive “VOICE MAIL” from computer to telephone or portable phone.

- Send and receive E-MAIL.

- Send and receive FAX.

- Receive DTMF.

## I.4 PCCOM RemoTeleCom

The PCCOM Remote TeleCom is an OCX programming development tool for made application software. That will use to enhance PCCOM TeleCOM functions (Pager, E-mail, Voice mail, Fax) through Internet or Intranet by stand-alone program or by browser through World Wide Web (WWW) under Windows 95/98/2000/NT/ME/XP. User may call OCX functions to communicate with Remote devices both Server and Client using Internet communication; or encapsulate OCX function and remote ActiveX Control, then run development application program under Internet browser (IE and Netscape).

## I.5 PCCOM SCOPE

PCCOM SCOPE is a software that monitors all the data and signals between two interacting devices, or a single device. With PCCOM SCOPE you can transform your PC into a total RS-232 analyzer without the need of expensive hardware or plug in boards. PCCOM SCOPE creates an unsurpassed tool for RS-232 device driver, communications protocol and traffic analysis.

## I.6 REMOTE CONTROL SOFTWARE

We also provide other hardware with complete Internet/Intranet remote control software tool for more function availability

Decision Industrial Interface

The Decision Industrial Interface was created to provide a standard way to access the functionality provided by all data acquisition products.

Decision Industrial Control

Develop a program controlling the Industrial digital and analog cards remotely at any part of the world using the Internet technology! Done it in your application program or via Internet browser! It extends Decision Industrial Interface from single computer development environment to client-server development environment.

Remote Voice

Long distance call will not be expensive as before. There is no cheaper way to talk to your friend from another country than to talk through Internet! And the good news is that you can develop your own software with Remote sound!

Remote Chat

This development tool is used for chatting / communicating with another computer in the Internet / Intranet.

Remote White Board

This development tool uses client – server remote drawing board, every client can shared their image drawn on server drawing board.

Remote Image Capture

This development tool allows you to capture images taken from the CCD camera via Internet.

Pinger

This development tool make it possible for you to make a program that check if your still connected to the network or internet and check the speed of the response of your connection!

Internet Location Server

This development tool is used to find an IP address of a particular client / computer in the Internet by just specifying its email address!

Remote Access Server

This development tool use to dial-up like and phone dialer technology, able to dial an ISP and monitor some of some IP address on network

APPENDIX J

# WARRANTY INFORMATION

## J.1 Copyright

Copyright DECISION COMPUTER INTERNATIONAL CO., LTD All rights reserved. No part of PCCOM software and manual may be reproduced, transmitted, transcribed, or translated into any language or computer language, in any form or by any means, electronic, mechanical, magnetic, optical, chemical, manual, or otherwise, without the prior written permission of DECISION COMPUTER INTERNATIONAL CO., LTD.

Each piece of PCCOM package permits user to use PCCOM only on a single computer, a registered user may use the program on a different computer, but may not use the program on more than one computer at the same time.

Corporate licensing agreements allow duplication and distribution of specific number of copies within the licensed institution. Duplication of multiple copies is not allowed except through execution of a licensing agreement. Welcome call for details.

## J.2 Warranty Information

DECISION warrants that for a period of one year from the date of purchase (unless otherwise specified in the warranty card) that the goods supplied will perform according to the specifications defined in the user manual. Furthermore that the PCCOM product will be supplied free from defects in materials and workmanship and be fully functional under normal usage.

In the event of the failure of a PCCOM product within the specified warranty period, DECISION will, at its option, replace or repair the item at no additional charge. This limited warranty does not cover damage resulting from incorrect use, electrical interference, accident, or modification of the product.

All goods returned for warranty repair must have the serial number intact.  Goods without serial numbers attached will not be covered by the warranty.

Transportation costs for goods returned must be paid by the purchaser.   Repaired goods will be dispatched at the expense of PCCOM.

To ensure that your PCCOM product is covered by the warranty provisions, it is necessary that you return the Warranty card.

Under this Limited Warranty, DECISION's obligations will be limited to repair  or replacement only, of goods found to be defective as specified above during the warranty period. DECISION is not liable to the purchaser for any damages or losses of  any kind, through the use of, or inability to use, the PCCOM product.

DECISION reserves the right to determine what constitutes warranty repair or replacement.

Return Authorization: It is necessary that any returned goods are clearly marked with an RA number that has been issued by DECISION. Goods returned without this authorization will not be attended to.